SSoW: Elumatec KS101 – V-Notch & Notching Saw

(Serial: 30750, Year: 1991)

This Safe System of Work applies to the Elumatec KS101 V-Notch and Notching Saw, used for creating accurate V-notches and angled cuts on PVC and aluminium profiles. It covers safe setup, clamping, cutting, blade control, and maintenance.

Required Competence & Training

Operators must:

- Be trained specifically on Elumatec KS-series saws

- Understand blade hazards and correct feed techniques

- Know how to adjust saw angles, fences, and stops

- Be competent with compressed air systems (if applicable)

- Understand emergency stop and isolation procedures

- Have manual handling awareness

Trainees must be supervised.

PPE Requirements

- Safety glasses (mandatory)

- Hearing protection

- Cut-resistant gloves for handling profiles (not during cutting)

- Safety footwear

- No loose clothing, jewellery, or unrestrained long hair

Pre-Use Checks

Before operation, the operator must check:

- Blade condition: sharp, undamaged, correct type installed

- Blade guard in place and functional

- Emergency stop operational

- Clamps engage and hold securely

- Angle settings and stops calibrated

- Extraction connected and working

- Air supply (if pneumatic) is stable and leak-free

- Work area clear of off-cuts

Report defects immediately.

Safe Operating Procedure

Start-up

- Complete all pre-use checks.

- Turn on main power and extraction.

- Select correct angle / V-notch setting.

- Position profile against fence and clamp securely.

- Keep hands clear of blade travel path.

Cutting Operation

- Allow saw to reach full speed before feeding.

- Never reach near the blade or clamp during cutting.

- Use steady, controlled feed and do not force saw through material.

- Stand to one side of cutting line to avoid kickback.

- Ensure clamp remains engaged throughout the cut.

- When cutting is complete, allow blade to stop before unclamping.

- Inspect finished cut for accuracy before proceeding.

Handling of Material

- Support long profiles to prevent sagging and twisting.

- Use team lifts for heavy or awkward sections.

- Avoid placing hands between profile and clamps.

Blade Change / Maintenance

Competent authorised personnel only.

- Apply LOTO and isolate saw.

- Wait for blade to fully stop.

- Remove guard using approved tools.

- Remove blade with correct spanner and gloves.

- Install new blade, check alignment and run-out.

- Refit guard and test run unloaded.

6. Key Hazards & Controls

| Hazard | Risk | Controls |
| --- | --- | --- |
| Contact with rotating blade | Amputation, deep cuts | Guarding, training, PPE |
| Material kickback | Impact injuries | Secure clamping, correct feed rate |
| Noise | Hearing damage | Mandatory hearing protection |
| Flying debris | Eye injury | Impact-rated eye protection |
| Manual handling | Strains | Team lifts, supports |
| Dust | Respiratory/eye irritation | Extraction, housekeeping |

Emergency Procedures

- Press Emergency Stop immediately if required.

- Isolate machine in case of abnormal noise, vibration, or blade breakage.

- Seek first aid for any injury.

- Record and report incidents to supervisor.

- Follow evacuation procedures during fire alarm.

Environmental Controls

- Use extraction at all times to control dust.

- Dispose of PVC and aluminium off-cuts in correct recycling bins.

- Keep saw area tidy, regularly remove debris.

- Report leaks or spills immediately.

Post-Use & Shutdown

- Allow final cut to finish fully.

- Wait for blade to stop before unclamping.

- Turn off the saw, then extraction.

- Clean saw bed and remove off-cuts.

- Inspect blade and report wear/damage.

- Store tools correctly.