SSoW: Pegas 230×280 GH-LR – Metal Horizontal Band Saw

(Serial: 2300227959, Year: 2022)

This Safe System of Work covers the safe operation of the Pegas 230×280 GH-LR horizontal metal band saw, used for cutting aluminium, steel, and other metals. It includes safe setup, cutting, blade tensioning, coolant use, and shutdown procedures.

Required Competence & Training

Operators must:

- Be trained specifically on the Pegas horizontal band saw

- Understand hazards relating to rotating blades and moving arms

- Know how to tension and align blades correctly

- Understand coolant and lubrication systems

- Be competent in manual handling of metal stock

- Be aware of entanglement, projection, and crush hazards

Trainees must be supervised.

PPE Requirements

- Safety glasses or face shield

- Hearing protection

- Cut-resistant gloves (for handling metal stock only; NOT during cutting)

- Steel toe safety footwear

- No loose clothing, jewellery, or unrestrained long hair

Pre-Use Checks

Operators must inspect:

- Blade condition: cracks, wear, missing teeth

- Blade tension correctly set

- Guide rollers aligned and undamaged

- Clamping vice fully operational

- Coolant level sufficient

- Coolant delivery system flowing correctly

- Emergency stops functional

- Work area free from swarf and metal off-cuts

Any defect must be reported and the machine isolated.

Safe Operating Procedure

Start-up

- Conduct all pre-use checks.

- Switch on main power supply.

- Adjust cutting angle (0°–60°) if required.

- Secure the workpiece in the vice, no hand holding.

- Confirm blade guard is in place.

- Stand clear of blade descent path.

Cutting Operation

- Start coolant flow (if applicable).

- Start the saw, let the blade reach full speed.

- Lower the cutting arm gradually, never drop it.

- Keep hands away from blade zone and vice.

- Do not adjust the workpiece while the blade is moving.

- Monitor for blade wandering or vibration.

- Allow the cut to complete fully before raising the arm.

Handling Metal Stock

- Use mechanical aids or two-person lifting for long/heavy bars.

- Support long sections to prevent tipping or sideways loading.

- De-burr cut metal edges using tools, not hands.

Blade Change / Adjustment

Only trained personnel.

- Apply LOTO.

- Ensure blade is completely stopped.

- Release blade tension slowly.

- Remove blade using cut-resistant gloves.

- Fit new blade ensuring correct tooth direction.

- Reapply tension and test-run unloaded.

Key Hazards & Controls

| Hazard | Risk | Controls |
| --- | --- | --- |
| Contact with moving blade | Amputation, deep cuts | Guarding, vice use, training |
| Kickback of metal bar | Impact injuries | Secure clamping, stable support |
| Coolant/floor contamination | Slips | Absorb spills, maintain flooring |
| Flying metal chips | Eye injury | Eye protection, extraction |
| Noise | Hearing loss | Hearing protection |
| Manual handling | Strains | Mechanical aids, team lifting |

Emergency Procedures

- Use Emergency Stop immediately if safety compromised.

- In case of blade breakage: stop machine, isolate, clear area.

- Seek first aid for any cuts or impact injuries.

- Report all incidents to supervisor.

- Follow site evacuation procedures if fire alarm or major fault occurs.

Environmental Controls

- Collect metal chips using magnetic sweeps or brushes.

- Dispose of metal off-cuts in scrap recycling bins.

- Prevent coolant spills, use drip trays and mats.

- Store coolant concentrate safely and mix according to COSHH requirements.

Post-Use & Shutdown

- Allow blade to stop completely.

- Raise cutting arm and clean saw bed.

- Turn off machine and coolant pump.

- Remove swarf using brush (never hands).

- Check coolant level and refill if needed.

- Report wear or any maintenance issues.