SSoW: Pertici BS771 – Bead Saw with Digi Stop

(Serial: 14V114, Year: 2014)

This Safe System of Work covers the Pertici BS771 bead saw with Digi Stop, used for cutting glazing beads and small PVC/aluminium sections to precise length. It includes safe setup, length setting via Digi Stop, clamping, cutting, and basic maintenance.

Required Competence & Training

Operators must be trained and competent in:

- Use of the Pertici BS771 bead saw controls

- Setting and reading the Digi Stop length system

- Correct positioning and clamping of small sections/beads

- Hazards from rotating saw blades and kickback

- Emergency stop and isolation (LOTO) procedures

- Manual handling of bundles of beads and profiles

Trainees must be supervised.

PPE Requirements

- Safety glasses or face shield

- Hearing protection

- Cut-resistant gloves for handling beads/profiles (NOT while operating the saw)

- Safety footwear

- No loose clothing, jewellery, or unrestrained long hair

Pre-Use Checks

Before use, the operator must check:

- Saw blade condition: sharp, no cracks, chips, or missing teeth

- Blade guard present, moves freely, and covers blade correctly

- Emergency stop works when tested

- Clamping device grips bead securely and without slippage

- Digi Stop system powers on, zeroes correctly, and moves freely on the rail

- Extraction / dust collection connected and operating

- Work area and infeed/outfeed clear of off-cuts, swarf, and tools

Any defect → isolate the machine and report.

Safe Operating Procedure

Start-Up

- Complete pre-use checks.

- Switch on main power and extraction system.

- Power up Digi Stop and home/zero it as per instructions.

- Enter required cut length on the Digi Stop and move the stop to position.

- Check the stop is locked and cannot move during cutting.

Loading & Clamping

- Place bead/profile against the fence and push up to the Digi Stop.

- Ensure the piece is flat and fully supported.

- Engage clamp – never hand-hold material during a cut.

- Keep hands away from blade path.

Cutting Operation

- Allow the saw blade to reach full speed before engaging the cut.

- Bring the saw through the cut with a smooth, controlled motion – do not force it.

- Stand slightly to the side of the cutting line to avoid potential kickback.

- Hold the operating handle only – keep the other hand away from the cutting zone.

- Once the cut is finished, return the saw head to its rest position.

- Wait for the blade to stop rotating before unclamping or reaching near the blade.

- Remove cut piece and off-cut, stack safely, and repeat as required.

Short Piece / Small Off-Cut Control

- Use auxiliary stops or jigs if cutting very short beads.

- Never attempt to hold very small pieces near the blade by hand.

- Avoid cutting pieces so small they cannot be clamped securely.

Blade Change / Maintenance

Authorised and competent personnel only.

- Apply LOTO and fully isolate the saw.

- Wait for blade to stop completely.

- Remove blade guard using the correct tools.

- Loosen arbor nut and remove blade using gloves.

- Fit replacement blade (correct spec and diameter), check rotation direction.

- Tighten arbor nut and refit guard.

- Spin by hand to ensure free movement, then test-run unloaded.

Key Hazards & Control Measures

| Hazard | Risk | Controls |
| --- | --- | --- |
| Contact with rotating blade | Amputation, severe cuts | Guarding, clamping, training, no hand-holding |
| Kickback of bead/profile | Impact injuries | Secure clamping, correct feed, stand aside |
| Flying chips / swarf | Eye injury | Safety glasses/face shield, extraction |
| Noise | Hearing damage | Hearing protection |
| Manual handling of bundles | Strains/sprains | Correct lifting, team lift if needed |
| Entanglement | Clothing/hair pulled in | Fitted clothing, hair tied back |

Emergency Procedures

Hit Emergency Stop immediately if:

  - Material jams, blade snags, or unusual vibration/noise occurs

  - Any person is at risk of contact with the blade

Isolate the machine and do not restart until checked.

For injuries (cuts, impact, eye injuries):

  - Stop work, give first aid, report to supervisor, record incident.

In case of fire or electrical fault:

  - Isolate power, raise alarm, follow site evacuation procedure.

Environmental Controls

- Use extraction to minimise PVC/aluminium dust accumulation.

- Collect off-cuts and swarf frequently – keep floor and bench clear.

- Dispose of waste in appropriate recycling or general waste containers.

- Clean up any oil/air leaks and report them.

Post-Use & Shutdown

- Finish last cut and return saw head to rest.

- Allow blade to come to a complete stop.

- Turn off saw and then the extraction, if permitted.

- Power down the Digi Stop system.

- Remove off-cuts, sweep/brush the bench and floor (no hands near blade).

- Check blade and clamp for obvious damage; report if found.

- Leave the area tidy and free from trip hazards.