SSoW: Pertici FC106 – Copy Router

(Serial: 01P103, Year: 2001)

This Safe System of Work covers the safe operation of the Pertici FC106 Copy Router, used for routing, slotting, and creating cut-outs in PVC and aluminium profiles. It includes loading, clamping, routing operations, bit changes, and housekeeping.

Required Competence & Training

Only authorised operators who have been trained in:

- Use of Pertici routing equipment

- Understanding router bit types, speeds, and cutting pressures

- Safe clamping and profile positioning

- Manual handling of long profiles

- Use of compressed air systems and foot pedals

- Awareness of entanglement and ejection hazards

Trainees must be supervised closely.

PPE Requirements

- Safety glasses or full face shield

- Hearing protection

- Cut-resistant gloves for handling material (not during routing)

- Safety footwear

- Close-fitting clothing; no loose sleeves, jewellery, or watches

Pre-Use Checks

Operators must complete the following:

- Ensure guards and protective screens are in place

- Check emergency stops

- Confirm foot pedal operation is working

- Inspect router bits for wear or cracks

- Ensure the profile clamping mechanism is functioning

- Test compressed air supply

- Verify depth stops, guides, and templates are set correctly

- Clear work area of debris and off-cuts

Report any defects before use.

Safe Operating Procedure

Start-up

- Complete pre-use checks.

- Select correct routing template or guide.

- Secure profile firmly in the clamps.

- Confirm the bit is appropriate for material type (PVC/aluminium).

- Turn on extraction (if fitted).

- Power on router but keep hands away from the spindle.

Routing Operation

- Use both hands on the handle/control for stable routing.

- Keep hands clear of the cutter at all times.

- Do not adjust the profile or clamps while the spindle is running.

- Track the template smoothly—avoid forcing the router.

- Allow the machine to reach full speed before cutting.

- Stay clear of potential ejection path of chips or debris.

- Maintain balance and firm footing to avoid slips during routing.

Handling Profiles

- Use teamwork or mechanical assistance when handling long or heavy profiles.

- Do not place fingers between clamps or guide rails.

Bit Change / Maintenance

Only competent personnel.

- Isolate machine using LOTO.

- Wait for spindle to fully stop.

- Remove router bit using correct spanner/tool.

- Install new bit and tighten securely.

- Complete a short test cut on scrap material.

Key Hazards & Control Measures

| Hazard | Risk | Controls |
| --- | --- | --- |
| Contact with rotating cutter | Serious lacerations, amputation | Guards, training, correct handling |
| Profile ejection | Impact injuries | Correct clamping, correct bit, steady operation |
| Noise | Hearing loss | Mandatory ear protection |
| Dust and chips | Eye injuries | Safety glasses/face shield, extraction |
| Manual handling | Strains, sprains | Team lifts, correct technique |
| Entanglement | Clothing/hair pulled in | Close-fitting clothing, tie back hair |

Emergency Procedures

- Activate Emergency Stop if an unsafe condition occurs.

- Isolate machine if smoke, vibration, or mechanical failure is detected.

- Seek first aid for cuts or impact injuries.

- Report all incidents to supervisor.

- Follow site evacuation if fire alarm sounds.

Environmental Controls

- Use extraction to reduce dust and airborne particles.

- Collect PVC/metal waste and dispose correctly.

- Keep work area clean to prevent slips and contamination.

Post-Use & Shutdown

- Allow spindle to stop fully before leaving the machine.

- Switch off router and compressed air.

- Remove off-cuts and waste.

- Clean machine and surrounding area.

- Store templates and tools safely.

- Report wear or defects.