SSoW: Reiplinger BSD-R6.5-SH-GL30 – Hydropneumatic Punch Tool

(Serial: 191002/0225/PP) (Serial: 14/10/6112) (Serial: 191104/0269/PP)

This is a hydropneumatic punch unit designed to crop/punch holes, slots or end features in PVC and aluminium profiles using a high-force air–oil cylinder. These tools typically operate with foot pedal or two-hand control, depending on configuration.

This Safe System of Work covers:

- Feeding PVC/aluminium profiles into the Reiplinger punch

- Actuating the hydropneumatic cylinder

- Removing material and cleaning the punch station

- Safe handling of punched profiles

- Clearing jams and performing basic maintenance

Required Competence & Training

Operators must:

- Receive training on Reiplinger punching equipment

- Understand hydropneumatic forces & high-pressure hazards

- Understand correct profile orientation for the punch die

- Know the hazards from die pinch points and trapped-energy release

- Be trained in emergency stop and pneumatic isolation

- Be competent in safe manual handling of long profiles

Trainees must be supervised.

PPE Requirements

- Safety glasses

- Cut-resistant gloves for handling material (NOT during punching)

- Safety footwear

- Tight-fitting clothing — no loose items or jewellery

- Hearing protection if used in noisy workshop environments

- Long hair tied back

Pre-Use Checks

Before use, check:

- Guarding around punch/die is present and secure

- Emergency stop works correctly

- Pneumatic hoses & connectors free from leaks

- Hydraulic reservoir (if visible) at correct level

- Punch die is intact, sharp and properly seated

- Waste ejection path clear

- Foot pedal or activation system functioning correctly

- Profile supports and stops in the correct position

Do not use the machine if any defect is found.

Safe Operating Procedure

Start-Up

- Perform all pre-use checks.

- Open air valve and charge the hydropneumatic system.

- Cycle the punch once (no profile) to ensure smooth motion.

- Prepare the workspace and ensure no obstructions.

Feeding the Profile

- Support profile along its full length.

- Keep hands at least 100mm from the punch die at all times.

- Position profile against backstop or alignment guide.

- Confirm correct orientation for the required punching operation.

Punching Operation

- Check hand positioning — keep completely clear of punch zone.

- Activate punch via:

  - Foot pedal, or

  - Two-hand safety controls
(depending on machine configuration)

- Allow the cylinder to complete full punch and full retraction.

- Do NOT try to pull or adjust material mid-cycle.

- Once punch retracts fully, remove profile and inspect.

Safe Removal of Waste

- Use pliers or brushes, never fingers to remove waste from die area.

- Dispose of cropped waste safely into designated bins.

Clearing Jams

If the punch jams or does not retract properly:

- Hit Emergency Stop immediately.

- Shut off air supply (bleed residual pressure if fitted).

- Only when system is fully depressurised, remove jammed material.

- If jam cannot be cleared safely → call maintenance.

Key Hazards & Control Measures

| Hazard | Risk | Controls |
| --- | --- | --- |
| Hydropneumatic punch force | Amputation/crush injury | Guards, safe distances, trained operators |
| Unexpected actuation | Serious injury | Foot pedal covers, two-hand controls, E-stop |
| Sharp punched edges | Cuts | Gloves for handling output |
| Flying fragments | Eye injuries | Safety glasses |
| Hose failure | Whiplash/impact | Regular hose inspection |
| Manual handling | Strains | Team lifting for long profiles |

Emergency Procedures

- Activate Emergency Stop immediately if unsafe condition occurs.

- Isolate air supply immediately after any malfunction.

- For injuries:

  - Stop machine

  - Provide first aid

  - Notify supervisor

  - Record incident

- For mechanical failure:

  - Isolate

  - Tag out

  - Report to maintenance

- For fire alarm:

  - Isolate air

  - Evacuate using site procedure

Environmental Controls

- Deposit PVC/aluminium waste into correct recycling bins.

- Keep area around foot pedal free from off-cuts.

- Report any air or hydraulic leaks.

- Keep working area clean to prevent slips and contamination.

Post-Use & Shutdown

- Complete final punch cycle.

- Turn off air supply and release residual pressure.

- Clean die area with brush.

- Remove waste and inspect for damage.

- Leave machine powered down and safe.

- Report any wear or defects.