SSoW: Stuga ZX-5 – Sawing & Machining Centre

(Serial: 243, Year: 2018)

This Safe System of Work covers the safe operation, cleaning, loading, unloading, and basic maintenance of the Stuga ZX-5 Sawing & Machining Centre used for cutting and machining PVC/Aluminium profiles in a production environment.

Required Competence & Training

Only authorised and competent operators may use this machine. Minimum requirements:

- Formal in-house training on Stuga ZX-5

- Understanding of emergency stop system

- Manual handling awareness

- Knowledge of safe isolation (LOTO) procedures

- Ability to interpret production drawings and measurements

- Understanding of pinch-points, cutting hazards, and CNC movement hazards

Trainees must be supervised at all times by a competent operator.

PPE Requirements

- Safety glasses (mandatory)

- Cut-resistant gloves for handling profiles (not when operating controls)

- Hearing protection

- Safety footwear (steel toe)

- Close-fitting clothing – no jewellery, loose sleeves or dangling items

Pre-Use Checks

Operators must inspect the machine before use:

- Check all guards and covers are in place and locked

- Confirm emergency stops function correctly

- Ensure extraction systems are operational

- Inspect saw blades for damage or excessive wear

- Check air supply pressure is within operating range

- Clear area of any off-cuts, swarf, or obstructions

- Confirm correct profile program is selected

Do not operate if any defect is found. Report immediately.

Safe Operating Procedure

Start-up

- Conduct all pre-use checks.

- Power on machine following manufacturer sequence.

- Ensure extraction is active.

- Load profile securely into the feed station.

- Select correct cutting/machining program.

- Stand clear of moving parts.

Operation

- Never bypass interlocks or guards.

- Keep hands clear of clamping zones.

- Allow the saw to complete each cutting cycle fully before loading new material.

- Monitor for unusual vibration, noise, or blade behaviour.

- Maintain clear communication with nearby operators.

- Do not attempt to remove off-cuts until all moving parts are fully stopped.

Loading & Unloading

- Lift profiles using proper manual handling technique.

- Keep fingers clear of clamps and feed rollers.

- Remove finished components only after the machine indicates the cycle is complete.

Safe Maintenance / Blade Change

Only trained personnel.

- Fully isolate power using LOTO.

- Release residual air pressure safely.

- Wait for saw blades to stop completely.

- Replace blade following manufacturer instructions.

- Dispose of worn blades in designated sharps bin.

Key Hazards & Control Measures

| Hazard | Risk | Control Measures |
| --- | --- | --- |
| Contact with moving blades | Amputation, cuts | Fixed guards, interlocks, trained operators, PPE |
| Profile ejection | Impact injury | Correct clamping, correct program setup |
| Noise | Hearing damage | Mandatory ear protection |
| Air pressure hazards | Bruising, impact | Ensure hoses & fittings are intact |
| Manual handling | Musculoskeletal injuries | Use mechanical assist where possible, correct lifting |
| Slips/trips | Injury from falls | Maintain clean work area, remove off-cuts |

Emergency Procedures

- Press Emergency Stop immediately for any unsafe condition.

- In case of injury:

  - Stop the machine

  - Inform supervisor

  - Provide first aid

  - Record incident

- In case of fire or electrical fault:

  - Isolate machine

  - Raise alarm

  - Follow site evacuation procedure

Environmental Controls

- Ensure dust extraction system is used during all cutting operations.

- Collect swarf and off-cuts and dispose in designated recycling containers.

- Prevent oil/air leaks and report environmental risks immediately.

Post-Use & Shutdown

- Allow machine to complete final cycle.

- Remove remaining profiles safely.

- Power down following manufacturer procedure.

- Clean surrounding area, remove swarf and debris.

- Report defects or maintenance needs.

- Return tools and equipment to storage.