SSoW: Urban AKS1110-450 – Single Head Welder

(Serial: 7823, Year: 2001)

This Safe System of Work covers the use of the Urban AKS1110-450 Single Head Welder used for welding PVC window and door profiles. It includes setup, operation, loading/unloading, safe welding practices, and shutdown procedures.

Required Competence & Training

Only trained and authorised operatives may use the machine. Operators must:

- Be trained specifically on Urban single-head welding equipment

- Understand temperature, pressure and welding cycle settings

- Know risk controls relating to hot surfaces and pinch points

- Understand emergency stop and machine isolation procedures

- Be trained in manual handling of window/door profiles

- Understand profile preparation requirements (clean, square, deburred)

Trainees must be supervised.

PPE Requirements

- Heat-resistant gloves for handling welded joints

- Safety glasses

- Safety footwear

- Long sleeves and close-fitting workwear

- No loose jewellery, watches or dangling clothing

Pre-Use Checks

Operators must carry out the following:

- Check welding plates are clean and free from contamination

- Confirm temperature settings match production specifications

- Inspect machine guards and safety covers

- Test all emergency stop buttons

- Ensure profile clamping mechanisms operate correctly

- Confirm air pressure is within operating limits

- Remove debris from work area

- Ensure extraction (if fitted) is running

Report defects immediately.

Safe Operating Procedure

Start-up

- Conduct all pre-use checks.

- Switch on main power and allow plates to reach correct temperature.

- Select the correct welding program.

- Position profiles securely in clamps.

Operation

- Do not place hands between welding plates at any time.

- Ensure the profile is correctly aligned before initiating cycle.

- Keep hands clear of clamps and moving parts.

- Allow welding cycle to complete fully, do not interrupt.

- Monitor welding bead and joint quality.

- Use both hands on designated handles when positioning profiles.

Weld Handling

- Welded joints and plates may be hot — use heat-resistant gloves.

- Allow welded frames to cool before stacking.

- Do not force or bend the welded joint while still warm.

Tooling / Plate Change

Only trained personnel.

- Isolate machine using LOTO.

- Allow plates to cool fully.

- Use correct manufacturer tools for removal.

- Reinstall plates and check alignment.

- Test on scrap profile before production.

Key Hazards & Control Measures

| Hazard | Risk | Controls |
| --- | --- | --- |
| Hot welding plates | Burns | Guards, PPE, trained operators |
| Moving clamps | Crushing, pinch injuries | Keep hands clear, interlocks |
| Incorrect temperature/pressure | Poor welds, rejects | Correct program, checks |
| Manual handling | Strains, sprains | Team lifts, correct technique |
| Electrical faults | Shock | PAT, isolations, inspections |

Emergency Procedures

- Press Emergency Stop if immediate danger.

- Isolate power in case of fire, smoke or electrical fault.

- Seek first aid for burns or crush injuries.

- Record all incidents with supervisor.

In a factory fire alarm:

- Stop work, isolate machine, follow evacuation route.

Environmental Controls

- Dispose of PVC off-cuts in recycling bins.

- Keep welding table clean to prevent contamination.

- Report any lubricant or air leaks immediately.

Post-Use & Shutdown

- Complete final welding cycle.

- Switch off heating plates.

- Power down via manufacturer procedure.

- Remove debris and clean work area.

- Store tooling safely.

- Report maintenance needs.