SSoW: Urban AKS1810-28/16 – Verti-Quad Welder

(Serial: 180113, Year: 2017)

This is a high-capacity quad-head PVC welding machine used for simultaneous multi-corner welding of window and door frames. Because of its complexity, this SSoW is written to fully cover multi-axis movement, hot plate hazards, and clamp sequencing.

This Safe System of Work applies to the operation of the Urban AKS1810-28/16 Verti-Quad Welder, including:

- Multi-corner PVC welding

- Frame positioning and clamping

- Plate heating controls

- Automatic cycle operation

- Safe handling of welded frames

- Tooling/plate maintenance

Required Competence & Training

Operators must:

- Receive full training on Urban quad-head welding systems

- Understand welding temperatures, pressure sequencing, and cycle times

- Be capable of aligning multi-piece frames correctly

- Understand hazards relating to hot plates, moving clamps, and automatic rams

- Know emergency stop, isolation (LOTO), and lockout procedures

- Be able to inspect weld quality and identify rejects

Trainees must be supervised at all times.

PPE Requirements

- Safety glasses

- Heat-resistant gloves (for removing newly welded frames)

- Safety footwear

- Close-fitting clothing

- Hair tied back, no jewellery or loose items

Pre-Use Checks

Operators must check:

- All hot plates are clean and contamination-free

- Guards, covers, and fixed guarding are securely in place

- Emergency stops functioning correctly

- All four clamps and movement axes operate smoothly

- Temperature and pressure settings match production requirements

- Air pressure stable and within correct limits

- Table and welding zones free from debris

- Extraction/ventilation active if fitted

Any defect must be reported immediately before use.

Safe Operating Procedure

Start-Up

- Complete all pre-use checks.

- Turn on electrical supply and heating plate system.

- Allow plates to reach set temperature.

- Select correct program for frame type and profile system.

- Position all frame sections in the quad clamps accurately.

- Confirm alignment before initiating cycle.

Welding Operation

- Keep hands away from clamps and plate approach zone at all times.

- Initiate automatic cycle once alignment is confirmed.

- Stand clear of closing clamps — do not lean into machine.

- Allow cycle to complete fully; do not override sequence.

- Observe plate insertion, melt cycle, and joining cycle safely from outside hazard zone.

- Once cycle finishes, wait for clamps to retract fully.

Handling Welded Frames

- Welded corners will be extremely hot — use heat-resistant gloves.

- Lift evenly from designated handling points to avoid twisting still-soft joints.

- Place on cooling rack; do not stack immediately.

- Inspect weld appearance and bead quality.

Plate Maintenance / Change

Authorised personnel only.

- Apply LOTO and fully isolate the machine.

- Allow plates to cool completely.

- Remove plate guarding and detach plate blocks.

- Clean or replace plates following manufacturer instructions.

- Reinstall plates, ensuring alignment and correct torque settings.

- Test plate movement using a dry cycle before production restart.

Key Hazards & Control Measures

| Hazard | Risk | Control Measures |
| --- | --- | --- |
| Hot plates | Severe burns | Guards, heat gloves, wait for cooling |
| Multi-head clamp movement | Crush injuries | Stand clear, automatic sequencing only |
| Misalignment of profiles | Rejects, rework | Visual checks before cycling |
| Unexpected machine motion | Impact | Emergency stop, guarding |
| Manual handling of frames | Strains | Two-person lifts for larger frames |
| Electrical/air failure | Fault risk | Isolation, inspections |

Emergency Procedures

- Press Emergency Stop for any unsafe condition.

- If a plate sticks or fails: isolate machine and do not reach inside.

- In case of burns: cool with clean running water; seek first aid immediately.

- Mechanical fault: isolate power and air.

- Follow site fire/evacuation procedures if alarm activates.

Environmental Controls

- Clean plates reduce smoke and unnecessary debris.

- Dispose of PVC off-cuts in recycling where available.

- Maintain ventilation in welding zone to manage fumes.

- Keep workspace free from tripping hazards and excess waste.

Post-Use & Shutdown

- Complete final welding cycle.

- Turn off hot plates followed by machine power.

- Clean welding table and remove PVC flash.

- Inspect plates and clamps for signs of wear.

- Store tooling blocks and alignment jigs correctly.

- Report all faults or maintenance needs.