SSoW: Urban AKS5500/5 – 5-Head Welder

(Serial: 550047, Year: 2021)

This is a high-output PVC welding system with five independently controlled welding heads, used for manufacturing windows and doors at scale. This SSoW reflects the increased mechanical movement, temperature control, and multi-axis hazards of a 5-head system.

This Safe System of Work covers safe operation of the Urban AKS5500/5 5-Head Welder, including:

- Multi-head PVC welding

- Profile alignment and clamping

- Hot plate operation

- Automatic welding cycle and synchronised head movement

- Handling of newly welded frames

- Tooling/plate changes and cleaning

Required Competence & Training

Operators must:

- Have full training on Urban 4- and 5-head welding systems

- Understand welding parameters (temperature, pressure, melt time, joining pressure)

- Know how to correctly set up 5-head clamps for various frame configurations

- Understand hazards from simultaneous movement of multiple heads

- Know LOTO procedures and all emergency stop points

- Be able to visually inspect weld quality and identify faults

Trainees must be supervised.

PPE Requirements

- Safety glasses

- Heat-resistant gloves (for removing freshly welded frames)

- Steel toe boots

- Close-fitting workwear (no loose clothing or jewellery)

- Long hair tied back

Pre-Use Checks

Operators must check:

- All hot plates clean and at correct temperature

- Guarding, fixed covers, and interlocks correctly positioned

- Emergency stops on all sides tested

- All five welding heads move freely and retract correctly

- Clamps engage and release evenly

- Air pressure within correct operating range

- Worktable free of debris, weld bead, and waste PVC

- Correct profile program selected on control panel

Report defects immediately.

Safe Operating Procedure

Start-Up

- Complete all pre-use checks.

- Turn on machine power and hot plate heating.

- Allow plates to reach specified temperature.

- Select correct welding program for the frame profile.

- Load and align frame sections into clamps.

- Verify alignment on all five heads before starting cycle.

Welding Operation

- Keep hands out of clamp zones and plate travel paths.

- Start automatic welding cycle using control panel.

- Stand clear of all five welding heads as they move simultaneously.

- Monitor the heating, melt, and joining phases through safe viewing areas.

- Never interrupt or override the automatic sequence.

- Wait for complete cycle and full retraction before touching the frame.

Handling Welded Frames

- Welds will be extremely hot – use heat-resistant gloves.

- Lift frames using two-person team for large items.

- Place on cooling racks; do not stack until welds have cooled.

- Check weld bead uniformity and join quality.

Tooling / Plate Change

Authorised maintenance personnel only.

- Apply LOTO.

- Allow plates to cool completely.

- Remove protective covers using correct tools.

- Replace or clean plates as required.

- Re-install covers and test head movement using a dry cycle.

- Document tooling changes in maintenance log.

Key Hazards & Control Measures

| Hazard | Risk | Controls |
| --- | --- | --- |
| Hot plates | Severe burns | Heat gloves, guards, temperature checks |
| Multi-head movement | Crush injuries | Interlocks, no entry during cycle |
| Misalignment | Weld failures, rejects | Visual checks before cycle |
| Unexpected restart | Injury | LOTO, warning signage |
| Manual handling | Strains/sprains | Team lifts, correct technique |
| Electrical/air failure | Machine fault | Daily checks, isolation |

Emergency Procedures

- Press Emergency Stop if any unsafe movement or failure occurs.

- For jammed plates or heads:

  - Stop machine, isolate, and call maintenance.

- In case of burns: cool under running water 20 minutes, seek first aid.

- For mechanical/electrical faults: isolate at main switch and report.

- Evacuate the building if fire alarm is triggered.

Environmental Controls

- Clean plates prevent smoke and contamination.

- Dispose of PVC waste in designated recycling streams.

- Keep welding area free of bead waste and trip hazards.

- Maintain ventilation to reduce fume build-up.

Post-Use & Shutdown

- Allow final welding cycle to complete fully.

- Turn off hot plates first, then main power.

- Clean welding table and remove scrap PVC.

- Inspect plates and clamps for wear.

- Store tools and blocks safely.

- Report defects/issues in maintenance log.