SSoW: Use of Air Compressor (Workshop Use)

(Generic workshop compressor – suitable for piston, screw or tank-based air systems)

This SSoW covers the safe operation of a workshop air compressor including:

- Start-up and shut-down

- Safe pressurisation

- Connecting/disconnecting air tools

- Draining condensate

- Inspection and housekeeping

- Dealing with hose failures, leaks or overpressure incidents

It applies to all fixed and portable compressors used to power pneumatic tools, croppers, welders, blow guns and cleaning nozzles.

Required Competence & Training

Operators must:

- Be trained in the safe use of workshop air compressors

- Understand hazards from compressed air, overpressure, and hose whip

- Know how to use pressure regulators and gauges

- Understand safe connection/disconnection of airlines

- Be trained in emergency isolation (air shut-off valves)

- Know compressor noise levels and PPE requirements

Trainees must be supervised.

PPE Requirements

- Safety glasses

- Hearing protection (compressors are >85 dB)

- Safety footwear

- Tight-fitting clothing (no loose sleeves near belts/pulleys)

- Gloves for handling hoses (not required for operation of tools unless specified)

Pre-Use Checks

Before turning the compressor on, check:

- Tank pressure reads 0 bar if starting from empty

- All guards in place over belts/pulleys (if belt-driven)

- Drain valve closed

- Oil level correct (if oil-lubricated unit)

- No air leaks from fittings, valves, or hoses

- Safety relief valve unobstructed

- Power cable/plug undamaged (for electric compressors)

- Adequate ventilation — do not use in confined areas

- Moisture trap not overfilled

- All hoses undamaged, no frays, cracks, or bulges

Do not use if any defect is found.

Safe Operating Procedure

Start-Up

- Check pre-use requirements.

- Ensure compressor is on stable ground and upright.

- Open main isolation valve (if fitted).

- Switch compressor on using control panel or switch.

- Allow tank to pressurise and cycle off automatically.

- Check gauges:

  - Tank pressure (receiver pressure)

  - Regulated output pressure

- Ensure pressure does not exceed safe working limit for tools attached.

Connecting Air Tools

- Set regulator to zero or minimum pressure.

- Hold airline securely and connect using quick-release fitting.

- Increase pressure slowly to tool requirement.

- Ensure hose runs are kept clear of walkways and sharp edges.

- Never kink or crush hoses.

General Use

- Keep hands and face away from connectors during pressurisation.

- Do not use compressed air to clean clothing or blow dust off skin.

- Never direct compressed air at another person — even low pressure can cause injury.

- Monitor compressor cycling – abnormal sounds indicate a problem.

- If overpressure occurs, safety valve must vent – evacuate area if continuous venting occurs.

Disconnecting Tools

- Turn regulator down to zero.

- Depressurise hose by running tool briefly or using bleed valve.

- Disconnect quick-release fitting firmly with both hands.

Tank Draining and Moisture Control

Daily (or after each shift):

- Switch off compressor.

- Open drain valve under tank slowly.

- Allow water/condensate to drain fully.

- Close valve and wipe area.

This prevents tank corrosion and internal rust.

Clearing Jams or Malfunctions

If a tool jams or hose bursts:

- Hit emergency stop (if fitted) or switch off compressor.

- Close main air isolation valve.

- Allow pressure to bleed off naturally or use designated dump valve.

- Replace damaged hose before restarting.

- Never attempt to repair pressurised lines.

Key Hazards & Controls

| Hazard | Risk | Controls |
| --- | --- | --- |
| Compressed air jet | Injection injury / eye injury | No blowing on skin, mandatory eye protection |
| Hose whip from burst | Impact injuries | Regular hose inspections, secure connections |
| Overpressure | Explosion / tank rupture | Relief valve, pressure gauge checks, maintenance |
| Noise | Hearing damage | Hearing protection |
| Hot surfaces | Burns | Avoid touching pump head/motor |
| Oil leaks (oil-lube models) | Fire/slip hazard | Daily inspection |
| Trip hazards from hoses | Falls | Good housekeeping, hose management |

Emergency Procedures

- Overpressure or relief valve failure:

  - Evacuate area

  - Isolate power

  - Call maintenance

- Hose rupture:

  - Step back

  - Isolate air supply

  - Replace hose

- Fire (compressor motor):

  - Disconnect electrical supply

  - Use CO₂ or dry powder extinguisher

  - Evacuate if unable to control

- Injury:

  - Provide first aid

  - Report incident and record in accident log

Environmental Controls

- Dispose of oily condensate via appropriate waste streams

- Prevent leaks from polluting drains or surfaces

- Keep ventilation areas clear to prevent overheating

- Maintain good housekeeping around compressor location

Post-Use & Shutdown

- Turn regulator down to zero.

- Switch off compressor power.

- Allow tank to cool and release pressure using bleed valve.

- Drain condensate.

- Coil hoses and store safely.

- Report defects or unusual noise/vibration.

- Ensure the compressor is left clean, isolated, and safe.